INDIAN INSTITUTE OF TECHNOLOGY ROORKEE

CANDIDATE’S DECLARATION

We hereby declare that the work that is being presented in this report entitled “DEVELOPMENT OF AC-DC BIDIRECTIONAL CONVERTER” in partial fulfilment of the requirement for the award of the degree of Bachelor of Technology in Electrical Engineering submitted to the Department of Electrical Engineering, Indian Institute of Technology Roorkee, INDIA is an authentic record of our own work carried out during the period from August 2013 to May 2014 under the guidance of  Dr. Pramod Agarwal, Professor, Department of Electrical Engineering, Indian Institute of Technology Roorkee.

The matter embodied in this project report has not been submitted by us for the award of any other degree or diploma.

Rahul Jangid

Nimit Jain

Mohit Tibrewal

Date: 
Place: IIT Roorkee

(B. Tech. EE)

CERTIFICATE

This is to certify that the above statement made by the candidates is correct to the best of my knowledge.

Dr. Pramod Agarwal

Professor
Department of Electrical Engineering IIT Roorkee

Date:
Place: IIT Roorkee

ACKNOWLEDGEMENT

We would like to express our sincere gratitude to Dr. Pramod Agarwal, Professor, Department of Electrical Engineering, Indian Institute of Technology Roorkee for their valuable guidance, support, encouragement and the inspirational support throughout the Project. We express deep and sincere sense of gratitude to all teachers of EE department for their encouraging and caring words and suggestions which have contributed towards completion of this project. We are indebted to all our classmates for taking interest in discussing our problems and encouraging us. We convey our deep sense of gratitude to the Head of Electrical Engineering Department (HOD), who directly or indirectly helped us during the work. Finally, we would like to express our deepest gratitude to the Almighty for showering blessings on us during the course of work.

Rahul Jangid

Nimit Jain

Mohit Tibrewal

(B. Tech. EE)

Date:
Place: IIT Roorkee

ABSTRACT

Bidirectional converter provides the ability to not only transfer energy from DC grid (battery) to AC grid (AC loads) but also transfers energy from AC grid (Wind Energy) to DC grid to charge the battery (energy storage). The battery to grid (B2G) mode allows power utility companies to offset peak power consumption thus allowing household consumers and industry corporations to save money on their electricity bills. This paper describes the design and provides the implementation details for a Three-Level PWM AC-DC bidirectional converter. Although the three-level PWM AC- DC design requires a more complex controller than its counterparts. The converter is realised by developing the bidirectional converter in MATLAB and its interconnection between AC grid and DC grid (battery).This interconnection is a crucial for current electricity distribution system because it enables connection of distributed energy resourced (DERs).

// TO DO : Write more stuffs here in paras

Microsoft visio – Ninja needs to download this.

CONTENTS // Will do at last

| Topic | Page no. |
| --- | --- |
| Candidate’s Declaration |  |
| Acknowledgement |  |
| Abstract |  |
| Contents |  |
| List of Figures |  |
| List of Table |  |
| List of Abbreviations |  |
| Chapter 1 : Introduction 1.1 Need of Bidirectional Convertor 1.2 Load Demand and BDC Overview 1.3 BDC Advantages |  |
| Chapter 2 : Methodology 2.1 Independent Inverter 2.2 Grid Tied Inverter 2.3 Grid Tied Rectifier 2.4 Grid Tied Charger |  |
| Chapter 3 : Hardware Implementation 3.1 Power Circuit 3.2 Control Circuit 3.3 Power Supplies |  |
| Chapter 4 : Experimental Setup And Results |  |
| Chapter 5 : Future Scope |  |
| References Appendix - A Appendix - B Appendix - C Appendix - D |  |

List of Figures

List of Tables

List of Abbreviations

Chapter -1

Introduction

# 1.1. Need for Bi-Directional Converters (BDC)

The design of power converter consumes time with a significant cost. Performance is generally determined after testing converters at nominal operating points. Thus, simulation can substantially reduce development cost. Using BDC we can reduce the number of converters in a grid used for different conventional resources like wind energy, solar energy, fuel cells etc. We can make two sub grids: AC grid and DC grid and connect these through a BDC.

# 1.2. Load Demand and BDC overview

Load demand is not constant throughout a typical day. In a typical 24 hour load profile, peak energy consumption is observed around 1:00pm and 11:00pm. At 1:00pm commercial loads consume around 3MW while household loads consume around 9MW. Around 11:00pm, 4MW & 11MW are consumed by commercial and household loads respectively. Energy consumption is at its lowest around 5:00am. Around this time loads are 2MW and 8MW. Figure 1.1 shows the daily load demand curve.

BDC proposes to smoothen out stressful load demands within the grid, especially in time slots when grid power is at its peak. It also provides ancillary service to alleviate grid consumption. BDC utilises battery as an energy source during peak time, and when the grid is of peak, the battery of DC grid is charged.

# 1.3. BDC advantages

They are suitable for peak shaving application. For this application, the batteries act as energy storage units. Energy is stored in the batteries during night time when the cost of electricity is at its lowest. During peak hours, unused energy are drawn from them to put power back to the grid. With this scheme, EV owners could produce revenue. They charge their batteries while electricity is cheap, and use them to put power back to the grid during peak hours. Utilities could also benefit from this by having increased system flexibility. They could use this as energy storage for intermittent renewable energy sources such as wind and solar. BDC is also suitable for regulating frequency fluctuations caused by system imbalances. Trough bidirectional converters, regulation is made possible by allowing the grid to absorb/release small quantities of energy from/to the batteries.

There are various advantages offered by DC grids to end-consumers, utilities and society, such as: improved energy efficiency, minimized overall energy consumption, reduced greenhouse gases and pollutant emissions, improved service quality and reliability, cost efficient electricity infrastructure replacement.

Technical challenges linked with the operation and controls of DC grids are immense. Ensuring stable operation during network disturbances, maintaining stability and power quality in the islanding mode of operation necessitates the improvement of sophisticated control strategies for DC grid’s inverters in order to provide stable frequency and voltage in the presence of arbitrarily varying loads. In light of these, the DC grid concept has stimulated many researchers and attracted the attention of governmental organizations in Europe, USA and Japan. Nevertheless, there are various technical issues associated with the integration and operation of DC grids.

# 1.4 Overall Bidirectional Topology

AC Grid Single Phase

Fig 1.2: Grid Power System

Chapter – 2

Methodology

In recent years, due to the growing concern over energy shortage and environmental pollution, the concepts of distributed generation (DG), and dc-ac based hybrid power systems have become progressively more popular with the decreasing costs of various clean renewable energy sources. Under small-scale power distribution system, such as house, commercial buildings, bi-directional single-phase PWM converters perform as key components to fulfill the following modes of operation.

Figure 2.1: Flow Diagram

# 2.1 Stand Alone Inverter Mode

When the grid is lost, the converter regulates the ac bus voltage and frequencies feeding the ac loads while the renewable energy sources or energy storage on the dc side to provide power. The ac side renewable energy resources would act as current sources.

# 2.2 Grid Tied Inverter Mode

When the grid is connected, the converter acts as a current source injecting or sinking power from the grid to balance the power flow between the dc and ac subsystems, while one of the dc resources regulates the dc bus voltage.

# 2.3 Grid Tied Rectifier Mode

When the grid is present, the converter regulates the dc bus voltage to sustain the dc loads while all dc side energy sources operate as current sources.

# 2.4 Grid Tied Charger Mode

When the grid is present, the converter charges the energy storage elements, such as batteries.

By applying the modeling methodology of semi-conductor switches, the switching, average and small signal models of the converter are derived under different modes of operation.

// to search more on these topics

Chapter – 3

Hardware Implementation of Single Phase Inverter

# 3.1 Power Circuit

Power circuit was fabricated using MOSFETs (IRF460) as basic switching devices. As discussed in previous chapters, standard H-bridge has been used. MOSFETs have been fitted on a module which receives the TTL gate pulses from pulse generator circuit and various components as discussed below performs the task of protection and isolation of devices.

## 3.1.1 Snubber Circuit:

It protects semiconductor devices by:

- Limiting device voltages during turn-off transients

- Limiting device currents during turn-on transients

- Limiting rate of rise of currents through semiconductor device at device turn-on

- Limiting rate of rise of voltages through semiconductor device at device turn-off

- Shaping the switching trajectory of the device as it turns on/off

Figure 3.1 – Snubber Circuit

Values of capacitance to be used can be calculated as under:

Cs=Imax*TminoffVin=27*168*10-924=0.1μF

## 3.1.2 Pulse Amplification and Isolation Circuit

Pulse amplification and isolation circuits first isolate the pulses coming from TTL sources from the power circuits and then pulses are amplified. Whole process of amplifying and isolation is carried out by MCT2E along with npn transistors and separate DC supply created using standard 12-0-12 transformer and voltage regulator-7812. Separate DC supply is must for each module as the pulses have to be given with respect to the source of each mosfet, hence we need separate ref. point for each pulse.

Figure 3.2 – Pulse Amplification Circuit schematic and its actual implementation

# 3.2 Control Circuits

Control Circuit has been made up of different components fabricated as separate PCB’s. Each PCB is a unique module in itself which performs a unique function. Various Components of control circuit have been discussed as under:

## 3.2.1 Voltage sensing circuit

Voltage sensing was performed using a standard AD-202 circuit as shown in figure below.AD-202 performs the task of isolation and stepping down of input signal. It was ensured that peak of voltage reaching the input terminals of AD202 is less than 5 volts. This was ensured by using a stepping down circuit using resistors.

## 3.2.2 SPWM Pulses Generation and deadband Module

This module has the function of generating uses following sub-modules:

- Triangular wave generation

- SPWM core circuit

### 3.2.2.1 Triangular Wave Generator

Since the required frequency has to be greater than 18Khz as per IEEE standards as mentioned in previous chapters, a minimum slew rate of 50V/us is required. Hence the op-amp used is a quad-opamp LF347n and a standard triangular wave generation circuit was followed to obtain a unity triangular wave of frequency 25 KHz.The circuit, schematic.and output has been shown in below figures.

Figure 3.2.2.1.1: Triangular Wave Generator

Figure 3.2.2.1.2: Triangular Wave output waveform

Peak to peak voltage output=(2*R4/R1)*Vsat

Frequency=R1R4*4*R2+R3*C1

### 3.2.2.2 SPWM Core Circuit

It compares the ref. and carrier signal to produce a CMOS logic output as per the switching strategy (unipolar or bipolar) as discussed in previous chapters. CMOS logics were converted to TTL logics using LM339.

## 3.2.3 Current Sensing Circuit

Current sensing circuit is fabricated using HALL Effect sensor, buffer and amplifier. The hall sensor is measuring the current by using the magnetic field around the conductor where the current is flowing through. The sensors are mostly internal compensated and support frequencies up to 200kHz with almost to losses. Therefore the hall sensor “LEM LA 125-P” will be used to measure the current in the test setup. This current transducer is able to measure currents up to 200A and 100kHz with an accuracy of 0.8% [15].

## 3.2.4 PI Controller Circuit (Analog)

# 3.3 Power Supplies

Here is the standard dual power supply using the Positive and Negative Voltage regulator ICs. It can give +12 volt and – 12 volt DC with a common ground. This power supply is ideal to power amplifier circuits that require well regulated dual power supply. It can give 1 ampere current to the circuit. 14-0-14 volt 1 Ampere step down transformer is used to drop 230 volt AC to 14 volt DC which is then rectified using the standard full wave bridge rectifier comprising D1 through D4.Smoothing capacitors C1 and C2 remove the ripples from low volt AC. Two regulator ICs are used to generate + 12 volt and – 12 volt DC. IC1 is 7812 positive regulator giving +12 volt regulated output.IC2 is 7912 negative regulator and its pins are slightly different from 7812. See the connection in the diagram. Regulated outputs from the regulator ICs are available from pin 3 which can be used to power the circuit. Capacitors (C3 and C4) act as noise filters to give clean DC.

7805 is a 5V fixed three terminal positive voltage regulator IC. The IC has features such as safe operating area protection, thermal shut down, internal current limiting which makes the IC very rugged. Output currents up to 1A can be drawn from the IC provided that there is a proper heat sink. A 9V transformer steps down the main voltage, 1A bridge rectifies it and capacitor C1 filters it and 7805 regulates it to produce a steady 5Volt DC. The circuit schematic is given below.

Figure 3.3.1: +- 12V Supply Circuit schematic [7]

Figure 3.3.2: +-5V Supply schematic

Chapter – 4

Experimental Setup and Results

Whole project was setup as per the strategies discussed in previous chapters. A connection diagram of overall project has been shown as under.

// INSERT CONNECTION DIAGRAM HERE

# 4.1 Hardware Design

// thesis chap 4 hardware design pg 22,

Figure 4.1.1: a) Unity rectifier, b) Unity inverter, c)Purely Capacitive, d)Purely Inductive [8]

# 4.2 Expected Results

Expected results pg 24

Figure 4.2.1: top) PWN Phase, mid) Phase-to-phase Voltage, bottom) Phase-to-neutral Voltage

# 4.3 Hardware Layout

Hardware layout 4.2

The proposed power electronic converter in the previous chapter is connected between the synchronous generator and the battery-bus. Therefore the semiconductors, gate drivers and cooling have to be designed according the specifications of these connected systems. Calculations have to be carried out on component ratings, gate drivers and thermal requirements, the outcomes have to be optimized in cost, weight and efficiency as stated in chapter 1.2. By summarizing chapter 2, the following hardware requirements can be presented:

- Topology is full bridge three phase converter with six active switches.

- Nominal phase current is 45 Arms.

- The system has to be able to operate at 2.2Inom in order to start the combustion engine.

- Nominal phase-to-neutral generator voltage is 251 Urms.

- The nominal operating frequency is 800 Hz.

- The inverter has to provide starting torque of 220 Nm, which is equivalent to 99 Arms.

- The battery-bus voltage is 600 V and its minimal value is 450 V (75%).

- The inductance and resistance of the generator are equal to 410 μH and 63 mΩ per phase respectively.

- Water cooling will be available, with the water temperature equal to 65°C.

// if less Gate driver

Pg 30 to be mailed by jangid make appendix on mosfet from that

# 4.4 Test Setup

Test setup

Before a prototype of the power electronic system can be made many aspects have to be investigated in practice. Therefore a test setup is developed to analyze the following things in more detail:

- Hardware component operating performance.

- Sensors output signals.

- IGBT operation.

- PWM methods.

By doing these measurements, the final design can be improved and the best PWM solution can be applied. Figure 4.16 shows the test setup of the power electronic converter which is connected to the synchronous machine.

# 4.5 Control Strategy (Carrier based Modulation)

5.2 chap in control strategy 5.9

Carrier based modulation is a voltage control method where a phase-to-phase voltage is generated based on a comparison between a carrier and a reference waveform. The carrier can be triangular or saw-tooth shaped, while the reference voltage is purely or edited sinusoidal. These two waveforms are generated and compared by a digital or an analog circuit and results in a PWM waveform. The choice between analog and digital depends on the application, but nowadays a digital controller is preferred because it is also able to handle other tasks (logging, control, communication) in its application.

## 4.5.1 Carrier Waveform

As mentioned above, the carrier can be saw-tooth or triangular shaped. During operation the reference voltage is compared to the carrier voltage, when the reference is higher than the carrier a positive pulse is generated and otherwise a zero pulse is the result.

Figure 4.5.1.1: PWM with Saw Tooth and triangular carrier [9]

For the triangular carrier only the trailing edge of the pulse varies when the value of the modulation index changes, in contrast to the triangular carrier, both sides of the switched output pulse from the phase leg are modulated which results in a symmetrical waveform (figure 5.4.). The advantage of the symmetrical waveform is that the odd harmonic sideband components around odd multiples of the carrier fundamental and even harmonic sideband components around even multiples of the carrier fundamental are eliminated [23]. Due to the bad harmonic performance of the saw-tooth carrier, only the triangle carrier will be used in this chapter.

Looking at one period of the reference voltage, the phase voltage with respect to the neutral (n) looks like in figure 4.5.1.2 (a):

The relationship between the reference and carrier frequency is called “frequency modulation ratio” and is defined as:

mf=fsf1

Where fs is the triangular carrier frequency and f1 is the fundamental frequency. The mf should be an odd integer in order to achieve an odd symmetry waveform and half-wave symmetry with the time origin. Therefore only odd harmonics are present and the even harmonics disappear from the phase to star point voltage (Ua0).

## 4.5.2 Reference Waveform

The output of the carrier based modulator can be controlled by changing the reference waveform which is compared with the carrier waveform. A sinusoidal fundamental output voltage can be achieved by using a sinusoidal reference. Output frequency can be determined and the effective phase-to-phase voltage amplitude can be controlled by changing the amplitude of the reference voltage, also called the modulation index (ma):

ma = Ûref /Ûtri

Where Ûref is the amplitude of the reference voltage, and Ûtri is the amplitude of the triangular voltage. Since the amplitude of the triangular carrier is 1, ma should be kept lower or equal than 1 to avoid over modulation and undesirable harmonics in the system [10] [11].

The characteristics of carrier modulation can be summarized as follows:

- Simple implementation (digital or analog).

- Fixed switching frequency.

- Losses and THD are directly related to the switching frequency.

Due to limited hardware in the test facility the measurements are carried out under the following conditions:

- DC bus voltage of 220V

- Maximum frequency 50Hz

- Closed loop control

# 4.6 THD and efficiency

Total harmonic distortion and converter losses were two important aspects in the decision of the PWM and control method. Based on these aspects, the carrier based modulation combined with the field oriented control method is chosen.

The proposed solution is implemented and total harmonic distortion and efficiency are measured. The measurements are done under the following conditions:

- Fundamental frequency: 50Hz

- DC bus voltage: 220V

- Closed loop control

- Inverter operation

As expected is the converter efficiency decreasing when higher switching frequencies are used and the dips in the THD are at odd frequency ratios with a multiple of 3. However, the losses are higher than expected and also the harmonic is much higher compared to the THD in figure 5.9. These differences can occur because of the following reasons:

- Unbalanced load

- Variations on the DC bus

- Current offset

- Incorrect PWM pattern

# 4.1 Voltage Sensing Module

Chapter – 5

Conclusion and Future Scope

# 5.1 Contribution and Conclusion

This thesis deals with the design and implementation of a bi-directional full bridge power electronic converter. The converter is able to rectify the voltage in generator operation and act as inverter during starting operation. The discussed solution is optimized in cost, controllability, efficiency and weight.

The hardware design of the full bridge converter consists of different components which are optimized according operating characteristics, price, efficiency and robustness:

- MOSFETs are used for the four switches in the design since they are available in the required high power ratings, provide relative low switching and conduction losses and support switching frequencies up to 200kHz.

- Opto-couplers are the best solution to provide electrical isolation between the control circuit and gate drivers since they are economic, compact and have a good performance. However, optical cables are used in the test setup because they are less sensitive for EMI compared to optocouplers, but are also more expensive.

- Hall sensors are used to measure the current. They provide an efficient and accurate current measurement with high electrical isolation, bandwidth and high frequency.

# 5.2 Future Work

This single-phase converter can be easily extended to a three phase converter by tripling the two-leg configuration and connecting the neutral points together.  This will open the possibilities of the following:

- Integration of DC microgrids to improve efficiency of energy storage and generation systems. For instance,a dedicated DC grid for electric vehicles in a city which also acts as energy storage system for the AC grid.

- Power sharing between isolated micro-grids through DC interconnection.

- Development of reliable, high quality grid may be feasible for small isolated industrial plants with both PV systems and wind turbine generator as the major power supply.

- The implemented software is limited to closed loop control. However, to control the power electronic converter, the proposed field oriented control loop needs to be implemented and tested.

- Another aspect are the opto-couplers. They are more economic and compact compared to the optic fiber cables. The switching behavior and requirements of these opto-couplers have to be investigated and a redesign has to be made in order to achieve a compact and robust solution.

References

[1]. Rashid. M.H, “Power Electronics circuits devices and applications”, PHI 3rd edition, 2004 edition, New Delhi.

[2]. Bimbhra .P.S "Power Electronics" Khanna Publishers, New Delhi, 2003. 4th Edition

[3]. S. Bose, Y. Liu, K. Bahei-Eldin, J.de Bedout, and M. Adamiak, “Tie line Controls in DC grid Applications,” in iREP Symposium Bulk Power System Dynamics and Control VII, Revitalizing Operational Reliability, pp. 1-9, Aug. 2007.

[4] R.H.Lasseter, “DC grids,”inProc.IEEE-PES’02, pp.305-308, 2002.

[5] Michael Angelo Pedraza and Ted Spooner, “A Survey of Techniques Used to Control  DC grid Generation and Storage during Island Operation,” in AUPEC, 2006.

[6] F. D. Kanellos, A. I. Tsouchnikas, and N. D. Hatziargyriou, “DC grid Simulation during Grid-Connected and Islanded Mode of Operation,” in Int. Conf. Power Systems Transients (IPST’05), June. 2005.

[7] Ming Li, Dong Dai & Xikui Ma, "Slow-Scale and Fast-Scale Instabilities in Voltage- Mode Controlled Full-Bridge Inverter" Journal: Circuits Systems and Signal Processing - CIRC SYST SIGNAL PROCESS , vol. 27, no. 6, pp. 811-831, 2008.

[8] J. Dixon, “Three-Phase controlled rectifiers”, Pontifcia Universidad Catolica de Chile, Chile.

[9] D. C. Rus, N. S. Preda, I. I. Incze, M. Imecs, Cs. Szabo, “Comparative Analysis of PWM Techniques: Simulation and DSP Implementation”.

[10] K. Zhou, D. Wang, “Relationship between space-vector modulation and three-phase carrier based PWM: A comprehensive analysis”

[11] N. Mohan, T. M. Undeland, W. P. Robbins (2003). Power Electronics. 3rd ed. Hoboken: Wiley. P225.

APPENDIX-A

A.1 IRFP-460 MOSFET specifications

A.2 AD-202, Isolation Amplifier Specifications